Atlas Copco Wuxi

# AIF															   9829225987

Ed. : 00

25-Sep-17

Product Type	:	ADSORPTION AIR DRYER

Product Range	:	X/XD-DESICCANT DRYER

Product Number	:	9829225987

# AHB - Component List X/XD1250-5000:

Note: The component symbols correspond with the numbers on the flow diagrams

| Component | Part name | Specifications |  | Where used |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | X ≤5000 | XD ≤5000 |
| CC1 / CC2 | Cooler | Duty: | To cool regeneration flow | YES, CC1 only | YES |
|  |  | Type: | Air/Water cooler |  |  |
|  |  | Spec: | Min. design temp: +1 °C |  |  |
|  |  |  | Max. design temp: +200 °C |  |  |
|  |  |  | Max. working press: 11 bar(g) |  |  |
| DESICCANT | Desiccant | Type: | Active Alumina | YES | YES |
|  |  | Duty: | To adsorb water / drying the air |  |  |
| OF1 | DDp outlet filter | Duty: | To avoid dust-particles > 1 µMin the air supply (at the outlet of the dryer) | YES, filter option | YES, filter option |
|  |  | Spec.: | Max. pressure is 16bar |  |  |
|  |  |  | Max. temperature is +66°C |  |  |
| PI01 / PI02 | Liquid Pressure Indicator | Duty : | To check visually the actual pressure in the vessel | YES | YES |
|  | vessel A / B | Spec : | Range: 0 to 16 bar(e) |  |  |
| PI-OF1 | Filter pressure drop indicator | Duty : | To check the pressure drop if filter replacement is required | YES, option | YES, option |
|  |  | Spec : | Range: 0 to 350 mbar |  |  |
| PT04 | Outlet | Spec : | Range: 0 to 17 bar(e) | NO | YES |
|  |  |  | -40°C to +135°C |  |  |
|  |  |  | Protection: IP67 |  |  |
|  |  | Material: | Main body : AISI304 stainless steel |  |  |
| PT01 / PT02 | Pressure vessel A / B | Spec : | Range: 0 to 17 bar(e) | YES | NO |
|  |  |  | -40°C to +135°C |  |  |
|  |  |  | Protection: IP67 |  |  |
|  |  | Material: | Main body : AISI304 stainless steel |  |  |
| PDT01 | Pressure Difference transmitter | Duty : | To measure the pressure difference between dryer inlet and outlet | YES | YES |
|  |  | Spec : | Range: 0 to 1000mbar |  |  |
| PDT02 | Pressure Difference transmitter | Duty : | To measure the pressure difference between heater, vessel and cooler. This is the protection of the heater | YES (For G dryer) | YES (For G dryer) |
|  |  | Spec : | Range: -50 to 200mbar |  |  |

| PDP01 | Pressure Dewpoint Transmitter | Duty : | To measure the pressure dew point at the dryer outlet | YES option | YES option |
| --- | --- | --- | --- | --- | --- |
|  |  | Spec : | Range: -100°C to +20°C |  |  |
|  |  |  | Operation Temperature: |  |  |
|  |  |  | -40°C tot +60°C |  |  |
| TT01 | Temperature hot air dryer inlet | Duty: | Measure the compressed air temperature at hot air inlet | NO | YES |
|  |  | Type: | PT1000 |  |  |
|  |  | Spec: | Range: -40°C to +270°C |  |  |
|  |  |  | Protection: IP54 |  |  |
|  |  |  | Max. pressure 50 bar |  |  |
|  |  | Material: | Housing : high grade steel |  |  |
| TT02 | Temperature heater outlet | Duty: | Measure/control the heated air | YES (For G dryer) | YES (For G dryer) |
|  |  | Type: | PT1000 |  |  |
|  |  | Spec: | Range: -40°C to +270°C |  |  |
|  |  |  | Protection: IP54 |  |  |
|  |  |  | Max. pressure 50 bar |  |  |
|  |  | Material: | Housing : high grade steel |  |  |
| TT05 | Temperature Cooler Inlet | Duty: | Measure the temperature of the air inlet of the cooler | YES | YES |
|  |  | Type: | PT1000 |  |  |
|  |  | Spec: | Range: -40°C to +270°C |  |  |
|  |  |  | Protection: IP54 |  |  |
|  |  |  | Max. pressure 50 bar |  |  |
|  |  | Material | Housing : high grade steel |  |  |
| TT06 / TT07 | Temperature Bottom Vessel A / B | Duty: | Measure the temperature of the bottom of the vessel A / B | YES | YES |
|  |  | Type: | PT1000 |  |  |
|  |  |  | Range: -40°C to +270°C |  |  |
|  |  |  | Protection: IP54 |  |  |
|  |  |  | Max. pressure 50 bar |  |  |
|  |  | Material: | Housing: high grade steel |  |  |
| TT12 | Temperature Dryer outlet | Duty: | Measure the temperature of the dryer outlet | YES | NO |
|  |  | Type: | PT1000 |  |  |
|  |  |  | Range: -40°C to +270°C |  |  |
|  |  |  | Protection: IP54 |  |  |
|  |  |  | Max. pressure 50 bar |  |  |
|  |  | Material: | Housing: high grade steel |  |  |
| TS01 | Overheating switch | Duty: Spec: Material: | Capillary switch to shut down the dryer in case of too high heater temperature | YES (For G dryer) | YES (For G dryer) |
|  |  |  | Range: 230°C to +650°C |  |  |
|  |  |  | Capillary: stainless steel |  |  |

| V1 / V2 | Inlet valve regeneration vessel A / B | Duty: | High performance Butterfly valve regeneration vessel A / B | YES | YES |
| --- | --- | --- | --- | --- | --- |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V3 / V4 | Outlet valves vessel A / B | Duty: | High performance Butterfly valve outlet vessel A / B | YES | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V5 | Regeneration bypass valve | Type: | High performance butterfly positioner valve | YES | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V6 / V7 | Depressure valve | Type | Angle valve | YES | NO |
|  |  | Spec | Design pressure 11 bar |  |  |
|  |  |  | Design temperature -10°C to +220°C |  |  |
| V8/V9 | Cooler outlet valves | Type: | High performance butterfly cooler outlet valve | YES | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V10 / V11 | Regeneration purge valve | Type: | High performance butterfly cooler outlet valve | YES | NO |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V13 / V14 | Regeneration valve vessel A / B | Type: | High performance butterfly valve(for XD) check valve(for X) between vessel and cooler | YES | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V16 / V17 | Cooler Air valves | Type: | High performance butterfly cooler air valve | NO | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V21 | Safety valve vessels A / B (option) | Duty: | To relief the overpressure in case of fire contingencies | YES, option | YES, option |
|  |  | Spec: | Opening pressure : 10.5 bar(e) |  |  |
|  |  | Material: | Body: brass |  |  |
|  |  |  | Valve cone: stainless steel |  |  |
|  |  |  | Spring: spring steel |  |  |
|  |  |  | Disc seal: Viton |  |  |
| V23 | Cooler valve | Type: | High performance butterfly cooler valve | NO | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| V24 / V25 | Cooling purge valve | Type: | Angle valve | YES | NO |
|  |  | Spec: | Design pressure 11 bar |  |  |
|  |  |  | Design temperature -10°C to +220°C |  |  |

| V27 | Manual valve | Type: | Manual ball valve | YES | YES |
| --- | --- | --- | --- | --- | --- |
|  |  | Spec: | Max. working pressure: 20 bar(e) |  |  |
|  |  |  | Working temperature: ≤170°C |  |  |
|  |  | Material: | Housing: brass, nickel plated |  |  |
|  |  |  | Ball: brass, nickel plated |  |  |
| V30 | Hot Air inlet valve | Type: | High performance butterfly valve hot air inlet | NO | YES |
|  |  | Spec: | Design pressure 11 bar PN16 |  |  |
|  |  |  | Design temperature 0°C to +180°C |  |  |
| R1/R2 | Main heater / Regulating heater | Duty: | To heat up regeneration air | YES | YES |
|  |  | Type: | Connection box : IP54 | (For G dryer) | (For G dryer) |
|  |  | Spec: | Voltage : 3ph 380V |  |  |
|  |  |  | Design pressure flange : 11bar |  |  |
|  |  |  | Min. Design temp. : -10°C |  |  |
|  |  |  | Max. design temp. : 220°C |  |  |
|  |  | Material: | hairpin rods AISI321 - diam. 12 |  |  |
|  |  |  | flange + connection box : painted steel |  |  |
| 101 | Dust Filter | Duty: | Dust filter for PDP sensor | YES | YES |
|  |  | Spec: | 0,1µm filter |  |  |
|  |  |  | Max. temperature: +66°C |  |  |
| 103 | Pilot air Filter | Duty: | To prevent dust entering pilot air | YES | YES |
|  |  | Spec: | 0,1µm filter |  |  |
|  |  |  | Max. temperature: +66°C |  |  |
| EWD1/2/3/4 | Electronic water drain | Duty: | Water Drain Coolers CC1 CC2 and vessel A/B with alarm contact | YES | YES |
|  |  | Spec. : | 230 V AC |  |  |
| 105 | 5/2 spool valve bistable | Duty: | To actuate the inlet valves | YES | YES |
|  |  | Spec.: | 24 V DC |  |  |
|  |  |  | design temperature: 0°C to + 50°C |  |  |
|  |  | Material: | Body: light alloy and cover in acetal resin |  |  |
|  |  |  | Internal parts : stainless steel, light alloy, acetal resin |  |  |
| 112 | Single direction flow restrictor | Duty: | To regulate the air flow through the PDP sensor | YES | YES |
|  |  | Spec : | Max. pressure: 30bar(e) |  |  |
|  |  |  | Temperature: -10°C to +70°C |  |  |

| Vessel A / B | vessel A / B | Type: | Pressure vessel | YES | YES |
| --- | --- | --- | --- | --- | --- |
|  |  |  | pressurized air internal, ambient air external |  |  |
|  |  | Spec: | design pressure: |  |  |
|  |  |  | 0 to 11 bar(e) (standard) |  |  |
|  |  |  | ambient temperature: -10°C to +50°C |  |  |
|  |  |  | design temperature: -10°C to +250°C |  |  |
|  |  |  | pressure variations : 22000 cycles |  |  |
|  |  |  | Corrosion allowance of non-stainless steel parts min. 1 mm |  |  |
|  |  | Material: | pressure vessel steel |  |  |
| STRAINERS | top and bottom vessel A / B | Duty: | Cylindrical strainer to keep desiccant in vessel | YES | YES |
|  |  | Material: | Stainless steel |  |  |
| SONIC NOZZLE |  | Duty: | To limit air speed through desiccant | YES, optional | YES, optional |